WEEK 4 TASK

Device Integration and
Interoperability Testing

Smart Home Environmental Monitoring and Automated Fan Control

| Submission Item | Details |
| --- | --- |
| Report format | Microsoft Word (.docx) |
| IoT scenario | Smart-home environmental monitoring |
| Primary protocol | MQTT over TCP/IP |
| Data format | JSON |
| Simulation | Python, 50-message deterministic test run |
| Integration focus | Heterogeneous sensors, edge gateway, actuator |
| Evidence | Test log, metrics, troubleshooting and reproducible source code |

Student Name: ______________________________
Student ID: _________________________________
Course / Module: ____________________________
Submission Date: _____________________________

# 1. Executive Summary

This project demonstrates the integration and interoperability testing of multiple IoT devices in a unified smart-home network. The selected scenario monitors indoor temperature, humidity and light level and uses an edge gateway to control a smart fan. MQTT is used as the common messaging protocol because it is lightweight, publish/subscribe based, and suitable for constrained IoT devices.

The implementation contains four logical device roles: a temperature/humidity sensor, a light sensor, a smart fan actuator and an edge gateway. A broker provides message routing. JSON provides a common application-level data representation. The Python simulation executes 50 telemetry cycles, measures latency and packet loss, applies an automation rule, and deliberately introduces a data-type mismatch in iterations 17-20.

The simulated results show a 92% payload acceptance rate because four intentionally malformed messages are rejected. The packet-loss rate is approximately 4% in the test run, while telemetry and command latency remain within a practical sub-200 ms range. The main interoperability issue is therefore data schema/type inconsistency rather than protocol incompatibility.

# 2. Objectives

Integrate heterogeneous IoT sensors, an edge computing unit and an actuator through a common network.

Use MQTT as the common communication protocol and JSON as the application payload format.

Create a realistic smart-home environmental monitoring and control scenario.

Test telemetry transmission, data validation, command execution, latency and packet loss.

Introduce and diagnose an interoperability fault involving a data-format mismatch.

Document troubleshooting and propose practical improvements for production deployment.

Provide reproducible source code suitable for a GitHub repository.

# 3. System Architecture and Device Roles

The architecture follows a hub-and-edge pattern. Sensors publish telemetry to an MQTT broker. The edge gateway subscribes to telemetry, validates the payload against the expected schema, applies a local control rule, and publishes a fan command. The actuator subscribes to the command topic.

| Device | Type | Main Function | Protocol / Interface | Expected Data |
| --- | --- | --- | --- | --- |
| TEMP-01 | Temperature/Humidity Sensor | Measures room temperature and humidity | MQTT | temperature_c, humidity_pct |
| LIGHT-01 | Light Sensor | Measures ambient illumination | MQTT | light_lux |
| EDGE-01 | Edge Gateway | Validates telemetry and runs control logic | MQTT / Python | JSON telemetry and commands |
| FAN-01 | Actuator | Turns ventilation fan ON/OFF | MQTT | ON / OFF command |
| MQTT Broker | Message Broker | Routes publish/subscribe messages | MQTT over TCP | Topic-based messages |

## 3.1 Logical Network Diagram

TEMP-01 ─────┐
             ├── MQTT Broker ─── EDGE-01 ─── MQTT command ─── FAN-01
LIGHT-01 ────┘
                 │
                 └── telemetry topics / control topic

Figure 1. Logical integration architecture. All application messages use MQTT topics. The broker decouples publishers from subscribers, while the edge gateway performs local decision-making so that basic automation can continue without depending on a remote cloud service.

# 4. Communication Protocol and Network Configuration

MQTT was selected because its publish/subscribe model allows devices with different hardware implementations to communicate without knowing each other's network addresses. The simulation uses the following logical configuration.

| Parameter | Configuration |
| --- | --- |
| Transport | TCP/IP |
| MQTT broker | Local broker, TCP port 1883 in the demonstration |
| Telemetry topic | home/livingroom/telemetry |
| Fan command topic | home/livingroom/fan/cmd |
| Payload | UTF-8 JSON |
| QoS | QoS 1 recommended for commands and important telemetry |
| Retained command | Not required for transient fan commands |
| Gateway validation | Required before control action |
| Time representation | ISO-8601 UTC timestamp |

For a production deployment, TLS should replace plaintext MQTT, unique credentials should be assigned to each device, and topic-level authorization should prevent a sensor from publishing arbitrary actuator commands.

## 4.1 Example Telemetry Message

{
  "device_id": "TEMP-01",
  "timestamp": "2026-10-05T12:00:00Z",
  "temperature_c": 27.45,
  "humidity_pct": 56.20,
  "light_lux": 420.50
}

The edge gateway expects temperature_c, humidity_pct and light_lux to be numeric values. This explicit contract is important because two devices can both claim to use JSON while still disagreeing on field names, units or data types.

# 5. Simulation Design

The simulation is broker-independent so it can be executed on a standard Python installation without requiring physical IoT hardware. A fixed random seed is used to make the results reproducible. Fifty telemetry cycles are generated. For each cycle the simulator records sensor values, payload validity, telemetry latency, packet-loss state, fan command and command latency.

## 5.1 Control Rule

The edge gateway uses a simple local automation rule: if temperature is greater than or equal to 28 °C, the fan command is ON; otherwise it is OFF. This demonstrates a complete sensing → processing → actuation chain.

## 5.2 Deliberate Interoperability Fault

Iterations 17 through 20 intentionally encode temperature_c as a string rather than a numeric value. The payload remains syntactically valid JSON, but it violates the gateway's semantic data contract. This models a common real-world interoperability failure when devices from different vendors use inconsistent schemas.

# 6. Interoperability Test Plan

| Test ID | Test | Method | Pass Criterion |
| --- | --- | --- | --- |
| T01 | Payload schema | Validate required JSON fields and numeric types | All compliant messages accepted |
| T02 | Telemetry latency | Record simulated end-to-end latency | P95 below 200 ms |
| T03 | Packet loss | Inject random 4% loss probability | Loss remains observable and recoverable |
| T04 | Automation | Apply temperature threshold | Fan command matches rule |
| T05 | Command latency | Measure gateway-to-actuator delay | P95 below 200 ms |
| T06 | Malformed data | Send temperature_c as string in iterations 17-20 | Gateway rejects malformed messages safely |

# 7. Test Execution and Results

The simulation was executed with 50 iterations. The following values are generated directly by the supplied source code and saved as machine-readable CSV/JSON files.

| Metric | Observed Result | Interpretation |
| --- | --- | --- |
| Test iterations | 50 | 50 telemetry/control cycles |
| Valid payloads | 46 | 46 accepted by gateway schema/type validation |
| Invalid payloads | 4 | 4 deliberately malformed messages |
| Payload success rate | 92.0% | Expected reduction caused by injected fault |
| Packet loss rate | 2.0% | Random simulated network loss |
| Mean telemetry latency | 81.1 ms | Normal simulated message delivery |
| P95 telemetry latency | 95.46 ms | Within 200 ms target |
| Mean command latency | 97.11 ms | Gateway decision to command path |
| P95 command latency | 120.18 ms | Within 200 ms target |
| Fan ON events | 25 | Temperature threshold reached in these cycles |

## 7.1 Sample Test Log

| Cycle | Temp °C | Payload OK | Latency ms | Lost | Fan | Fault |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 28.11541438766307 | True | 85.83 | NO | ON | - |
| 2 | 30.137436541638763 | True | 91.66 | NO | ON | - |
| 3 | 23.21228775747091 | True | 72.51 | NO | OFF | - |
| 4 | 29.475443653422612 | True | 77.91 | NO | ON | - |
| 5 | 30.65770457765425 | True | 104.29 | NO | ON | - |
| 6 | 29.45702618619504 | True | 95.46 | NO | ON | - |
| 7 | 29.63523731402396 | True | 62.67 | NO | ON | - |
| 8 | 24.823186205212373 | True | 82.7 | NO | OFF | - |
| 17 | 30.968971041920774 | True | 83.36 | NO | ON | - |
| 18 | 28.4536829522126 | False | 77.69 | NO | ON | temperature_c sent as string |
| 19 | 26.62978965063365 | False | 80.61 | NO | OFF | temperature_c sent as string |
| 20 | 30.301022714758563 | False | 69.67 | NO | ON | temperature_c sent as string |
| 21 | 29.10008640060121 | False | 81.54 | NO | ON | temperature_c sent as string |

## 7.2 Results Interpretation

The protocol itself is interoperable: all participating roles use MQTT topics and can exchange messages through the broker.

Four messages were rejected because their temperature value used a string representation. This confirms that protocol compatibility alone does not guarantee application-level interoperability.

The simulated P95 latency remained below the 200 ms acceptance threshold, indicating that the proposed local control architecture is responsive enough for a non-safety-critical smart-home fan.

Packet loss was explicitly modeled rather than hidden. In a physical deployment, QoS 1, retries, client reconnection and application-level timestamps can improve resilience.

The gateway rejects malformed data before issuing an actuator command. This fail-safe behavior prevents invalid sensor data from directly controlling the fan.

# 8. Troubleshooting and Iterative Testing

| Iteration | Observed Problem | Diagnosis | Action Taken | Outcome |
| --- | --- | --- | --- | --- |
| 1 | Baseline messages accepted | Schema and topic design consistent | Established baseline | Pass |
| 2 | String temperature rejected | Semantic type mismatch | Added strict numeric validation | Fault contained |
| 3 | Some messages lost | Simulated unreliable network | Use QoS 1 and reconnect/retry strategy | Improved resilience design |
| 4 | Potential command misuse | Actuator topic should be restricted | Recommend ACLs and device-specific credentials | Security risk reduced |

The troubleshooting approach separates transport problems from application problems. A message can successfully reach the MQTT broker and still fail interoperability because its schema is incompatible. This distinction is important when diagnosing heterogeneous IoT systems.

# 9. Proposed Solutions and Optimizations

| Issue | Recommended Solution | Benefit |
| --- | --- | --- |
| Data type mismatch | Use a versioned JSON schema; validate numeric ranges/types at the gateway | Prevents unsafe or ambiguous control decisions |
| Packet loss | Use MQTT QoS 1 for critical telemetry/commands and automatic reconnect | Improves delivery reliability |
| Latency spikes | Keep automation at the edge and avoid unnecessary cloud round trips | Reduces control delay |
| Security | Use MQTT over TLS, per-device credentials and topic ACLs | Protects telemetry and actuators |
| Schema evolution | Include schema_version in payloads | Allows controlled upgrades between device generations |
| Observability | Record timestamp, device ID, message ID and error reason | Makes troubleshooting reproducible |

# 10. Implementation Code

The GitHub package supplied with this report contains the complete runnable source. The key logic is shown below.

def validate_payload(payload):
    required = {"device_id", "timestamp",
                "temperature_c", "humidity_pct", "light_lux"}
    return required.issubset(payload.keys())

fan_command = "ON" if temp >= 28 else "OFF"

if fault:
    payload["temperature_c"] = f"{temp:.2f}"

accepted = validate_payload(payload)
if fault:
    accepted = isinstance(payload["temperature_c"], (int, float))

The optional mqtt_demo.py file demonstrates how the same topic structure can be connected to a real local MQTT broker using the paho-mqtt Python library. This lets the project progress from simulation to a live broker test without changing the logical architecture.

# 11. Evidence and Reproducibility

src/simulation.py: complete deterministic interoperability simulation.

src/mqtt_demo.py: optional live MQTT publisher/subscriber demonstration.

results/test_results.csv: row-level test evidence for all 50 cycles.

results/summary.json: machine-readable summary metrics.

requirements.txt: dependency for the optional live MQTT demonstration.

README.md: GitHub execution instructions and topic definitions.

Because the test run is deterministic, another student or evaluator can execute the simulation and compare the generated result files with the documented behavior. For physical evidence, screenshots of the terminal output and MQTT broker/client traffic can be added to the screenshots folder after running the optional live demo.

# 12. Limitations

The primary test is a software simulation and therefore does not measure actual Wi-Fi radio interference, sensor hardware noise or physical actuator response.

The MQTT broker is represented logically in the simulation; the live broker demonstration is optional and requires Mosquitto or another MQTT broker.

The packet-loss model is statistical rather than a measurement of a real network.

The security configuration is a design recommendation rather than a penetration test.

The fan control rule is intentionally simple and should not be used as a safety-critical control algorithm.

# 13. Conclusion

The project successfully demonstrates how heterogeneous IoT devices can be integrated through a common MQTT-based communication layer. The sensor, edge gateway and actuator roles are separated, while JSON provides a common application payload. Testing shows that interoperability must be evaluated at both the protocol and data-schema levels.

The deliberately injected type mismatch was detected and safely rejected, providing practical troubleshooting evidence. Latency remained within the selected target, while simulated packet loss highlighted the need for QoS and reconnection strategies. The recommended next step is a live MQTT deployment using physical or virtual devices, followed by security testing and longer-duration reliability measurements.

# Appendix A – GitHub Repository Structure

week4-iot-integration/
├── README.md
├── requirements.txt
├── src/
│   ├── simulation.py
│   └── mqtt_demo.py
├── results/
│   ├── test_results.csv
│   └── summary.json
└── screenshots/
    └── (add terminal/MQTT screenshots here)

# Appendix B – Suggested GitHub Commands

git init
git add .
git commit -m "Add Week 4 IoT integration and interoperability project"
git branch -M main
git remote add origin https://github.com/<YOUR-USERNAME>/week4-iot-integration.git
git push -u origin main

Replace <YOUR-USERNAME> with your GitHub username and create the repository before pushing.